STEREO

IMPACT

Boom Suite On-Orbit Power Cycle Procedure

input.doc

Version M – 2014-Jul-06

David Curtis, UCB IMPACT Project Manager

Date Run:____________________________________

Observatory:________________________________

# Document Revision Record

| Rev. | Date | Description of Change | Approved By |
| --- | --- | --- | --- |
| A | 2006-Jul-20 | Preliminary Draft | - |
| B | 2006-Nov-16 | Add options, default settings |  |
| C | 2006-Dec-15 | Add burst patch, SWEA spec tlm, PLASTIC SSR limit |  |
| D | 2007-Jun-15 | Update per latest software load (38/39) |  |
| E | 2008-Mar-4 | Update per latest software, add sequence loads, run cal. |  |
| F | 2009-Mar-24 | Add/update sequences 13-19 Change burst trigger defaults Add Code20090115_RAMPatch |  |
| G | 2009-May-8 | Add PatchNoSTEUV40 Return to default SWEA sweep, 1ev-2keV SWEA MCP -> 3150V Add BurstMode4 option |  |
| H | 2009-May-14 | Redlines from STB reset |  |
| I | 2009-Nov-5 | Add /ExecPolled to enable SWEA recovery |  |
| J | 2011-Jun-16 | Modify to accommodate V42/43 (remove patches now part of code) |  |
| K | 2014-Feb-25 | Modify to run out of EEPROM |  |
| L | 2014-Jun-25 | Modify IDPU startup (3.1) |  |
| M | 2014-Jul-6 | Correct |  |

# Distribution List

Dave Curtis, UCB

Peter Schroeder, UCB

Table of Contents

Document Revision Record	i

Distribution List	i

1.	Overview	1

1.1.	Introduction	1

1.2.	Document Conventions	1

1.3.	Applicable Documents	1

2.	Test Setup	2

2.1.	Test Equipment	2

2.2.	Startup GSE	2

3.	Power On	3

3.1.	Startup IDPU	3

3.2.	Startup MAG Heater	3

3.3.	MAG Check	3

3.4.	Configuration	4

3.5.	Startup SWEA/STE-D	6

3.6.	SWEA MCP Heater Adjust	6

3.7.	STE Adjustment	7

3.7.1.	SSWEA NRHV	8

3.7.2.	SWEA MCP HV	10

3.7.3.	SWEA MCP HV Ramp Up	10

3.7.4.	SWEA V0 Setting	11

3.7.5.	Run SWEA Calibration Sequence	11

3.8.	Final Configuration	12

4.	Power Off	14

4.1.	Power off SWEA HV	14

4.2.	STE Door Close	14

4.2.1.	STE-U Door Close	15

4.2.2.	STE-D Door Close	15

4.3.	SWEA Power Off	15

4.4.	IDPU Power Off	16

# Overview

## Introduction

The STEREO IMPACT Boom Suite consists of the Instrument Data Processing Unit (IDPU) plus the boom-mounted instruments (SWEA, STE-D, STE-U, and MAG).  The IDPU if the single point interface between these instruments and the STEREO spacecraft for telemetry.  The IDPU also interfaces to the SEP Suite and PLASTIC instruments.

This document describes the procedure for power cycling the boom suite on-orbit.

This proc should only be run when the spacecraft is in orbit.

## Document Conventions

In this document, TBD (To Be Determined) means that no data currently exists.  A value followed by TBR (To Be Resolved) means that this value is preliminary.  In either case, the value is typically followed by a code such as UCB indicating who is responsible for providing the data, and a unique reference number.

Commands to be typed into the C&T GSE (stgsect) are indicated as follows: /SystemNOP.  After typing the command hit the Enter key to send.

## Applicable Documents

The following documents are closely interrelated with this specification.  All documents can be found on the Berkeley STEREO/IMPACT FTP site unless otherwise indicated:

http://sprg.ssl.berkeley.edu/impact/dwc/

- 7381-9012B  (IMPACT/Spacecraft ICD, on the APL web page)

- ICD/IMPACT_CTM (command & telemetry database)

# Test Setup

## Test Equipment

- Command & Telemetry GSE connected to spacecraft emulator via TCP/IP

C&T GSE version date_____________

C&T Database version:__________, Date:________________

## Startup GSE

- Power up C&T GSE PC and start C&T GSE software (stgsect).

- On the C&T GSE PC start up the SSH client and connect to the MOC (at cho.jhuapl.edu; a profile should be set up to allow you to do that – see SSH.txt if not).   You will need a password.

- Connect C&T GSE to the MOC – push the TCP/IP next to the TLM display on the Received Data Panel:

Verify the TLM light comes on:____________

- Connect C&T GSE to MOC for commands – push the button next to the CMD display on the Received Data Panel:

Verify the CMD light comes on:____________

- On the C&T GSE push the Record button.  Determine the name of the telemetry file

File Name:__________________________________

- Enable the stgsect Telemetry Server.

# Power On

Record Start Time____________________

## Startup IDPU

- Request the spacecraft TC power-on the IDPU.   Record the bus voltage and IDPU current from the spacecraft displays

Bus Voltage _______________V

IDPU Current _____________mA

- Verify C&T GSE is getting telemetry.  On IDPU SOH panel, UTC time should correspond to time on Emulator PC (typically set to UTC), and should be incrementing every two seconds.

Verify C&T GSE getting telemetry _____________

- If you wish to remain in boot mode (because you want to load EEPROM or EEPROM image is corrupt), set the /SetBoot 128 command within 10 seconds of power-up (or send /RestartToPage 0 to reset the IDPU, then send /SetBoot 128 command within 10 seconds). Alternatively, if you don’t change SetBoot it defaults to zero, which will run the first EEPROM image out of RAM.  You can set SetBoot to 0-4 in the first 10 seconds to select any of the 4 EEPROM images to be run out of RAM.  Normally we run out of EEPROM since it is less prone to single event upsets.  See Step 4 to do that.

SetBoot _____________

- To run out of EEPROM, send /RestartToPage <N> at any time, where N indicates the EEPROM image you wish to run (N=8 for the first image, 9 for the second, 10 for the third, and 11 for the fourth.  As off June 2014 the latest image is the 4th, the the correct command is /RestartToPage 11.

N:_____________

- After 10 seconds ISoftwareVersion should change to correspond to the EEPROM software version number.  Verify no errors are reported and the version changes

Record value of ISoftwareVersion _____________

Record value of IERRORCount _____________

Record value of IBootSelect _____________

## Startup MAG Heater

- If the MAG heater is powered off, request the TC to power it on.  Record the MAG Heater current from the spacecraft displays

MAG Heater Supply Current_________mA

- Record the MAG temperature on the C&T GSE:

IMAGTemp__________C

- Record the telemetry MAG heater current (note that this is not expected to be an accurate measure of the actual current, but should correspond to the measurement made above to ~20%).  The MAG heater will not come on above 10C, but will take a ~5mA standby current.

IMAGHeater_________mA

## MAG Check

- If MAG starts up saturated (MAG values on MAG GSE all =1) , send /MAGEnableIF to restart interface

Y/N_________

## Configuration

- Clear the error by sending /ClearErrors.  IErrorCode should return to blank and IErrorCount should return to zero.

Verify__________

- Enable the instrument startup/shutdown sequences.  Start IMPACTPowerSeqEnable. Verify the IDPU SOH CmdSeqEnable housekeeping shows sequences 0-5 and 8-11 enabled.

Verify__________

- Upload the latest MAG zeros (used for SWEA PAD):

Start MagZeros.cmd

OR:

/SetMagZeros _________,_____________,___________

Verify__________

- Enable and start the MAG Calibration sequence:

  - /EnableCmdSequence 13

  - /StartCmdSeq 13

Verify__________

- Enable the SWEA Calibration sequence:

  - /EnableCmdSequence 14

Verify__________

- Enable the STE Calibration sequence:

  - /EnableCmdSequence 15

  - /EnableCmdSequence 16

  - /EnableCmdSequence 17

Verify__________

- Enable the STE Low Energy Calibration sequence:

  - /EnableCmdSequence 19

Verify__________

- If SEP is powered up but the interface is not enabled send:

  - /SEPEnableIF

Verify SEP Interface is Enabled

Verify__________

- If PLASTIC is powered up but the interface is not enabled send:

  - /PLASTICEnableIF

Verify PLASTIC Interface is Enabled

Verify__________

## Startup SWEA/STE-D

- Request the TC power-on the SWEA/STE-D service.  Wait 10 seconds, then record the current monitor on the spacecraft monitor

SWEA Current ________________mA

- Verify that on the C&T GSE ISCStatus display the SWEA Power display shows On (1), and on the Interface Active display, SWEAInterface shows active (1).  If not send /SWEAEnableIF

Verify___________

SWEA Current ________________mA

## SWEA MCP Heater Adjust

- Monitor the SWEA MCP Temperature for a while (in parallel with other activities).  Set the MCP heater to maintain an MCP temperature between –25 and +30C.  Within this range a cooler temperature is desirable, but leave margin to cover low bus voltage cases.  Use the command /SetSWEAMCPHeater to set the heater duty cycle between 0 and 10 (10=100%).  Default value = 0.  Nominal value for Obs A is 1, Obs B is 2.

| UTC Date/Time | Bus Voltage | Heater Setting | ISWEAMCP Temp | ISWEADAC Temp | SWEA Temp (S/C) | ISTED Temp |
| --- | --- | --- | --- | --- | --- | --- |

## STE Adjustment

- Start the STGSE-STE GSE (ste.exe).  Enable the C&T GSE telemetry server and connect the STE gse (TCP/IP button).  Verify that the Connect light comes on on the STE GSE.

Verify___________

- Display the STE Rates panel and wait for it to update.  Set to Raw Counts, so it reads counts per accumulation interval (2 seconds). Record the rates:

LLD	     ULD	   RESET

STEU Det0	_________	_________	__________

STEU Det1	_________	_________	__________

STEU Det2	_________	_________	__________

STEU Det3	_________	_________	__________

STED Det0	_________	_________	__________

STED Det1	_________	_________	__________

STED Det2	_________	_________	__________

STED Det3	_________	_________	__________

- If at ambient conditions (not vacuum, below +40C), send the following command to adjust the door timing:  start STEDoorAir.  If  in vacuum with STE temperatures below –35C do:  start STEDoorVacCold.  If in vacuum and between –35C and +20C do: start STEDoorVacWarm.  If in vacuum and above between +20C and +40C do: start STEDoorVacHot

STE-U temperature___________C

STE-D Temperature___________C

Script:__________________

- Start the STE Calibration sequence to test STE and set current nominal bias and threshold values:  /StartCmdSeq 15

Verify___________

- At the end of the STE Cal sequence verify the STE-D Door opens

Verify___________

STE Door Open time (Start/End):_______________________________

SWEA High Voltage Power On

TO BE PERFORMED ON ORBIT, NO SOONER THAN 1 HOUR AFTER SWEA COVER OPEN, TO AVOID DAMAGE TO SWEA MCP DETECTOR.

### SSWEA NRHV

The SWEA Non-Regulated High Voltage (NRHV) supply is used to generate the analyzer deflector and sweep supply voltages.  It operates at ~1500V, and is protected by a number of hardware and software protections against accidental turn-on.  Generally on the bench this supply is safe to be powered up.  However, ensure that the MCP is not enabled except in vacuum.

- Verify the SWEA door has been open for at least 1 hour.  If not, use 6.5 to open it.

Verify_____________

- Verify a high voltage enable plug is installed in SWEA-J2.

Enable Type___________

- Enable NRHV by sending the following two commands separated by no more than 15 seconds.

/SWEAArmNRHV

/SWEANRHVOn

Verify no commanding errors reported on the C&T GSE

Verify___________

- Verify that the SWEA NRHV state is On (on the C&T GSE ISWEASTEDDig display, NRHVENB = 1).

Verify___________

- Record the following high voltage values from the C&T GSE SWEA/STE-D Housekeeping display:

ISWEANR5V___________

ISWEAAnal___________

ISWEADefl1___________

ISWEADefl2___________

- Send /SWEAFHKPAnal to request a readout of the analyzer HV waveform.  Later use DECOM to display the waveform and append to this proc.  Verify three ApID 214 packets are sent.

Verify___________

- Send /SWEAFHKPDef1 to request a readout of the Deflector 1 HV waveform.  Later use DECOM to display the waveform and append to this proc.  Verify three ApID 214 packets are sent.

Verify___________

- Send /SWEAFHKPDef2 to request a readout of the Deflector 1 HV waveform.  Later use DECOM to display the waveform and append to this proc.  Verify three ApID 214 packets are sent.

Verify___________

- Send /SWEASampleFHKP 0  to switch the FHKP readout to MCP (reduces noise in STE-D).

Verify___________

SWEA MCP HV

TO BE PERFORMED ON ORBIT, NO SOONER THAN 1 HOUR AFTER SWEA COVER OPEN, TO AVOID DAMAGE TO SWEA MCP DETECTOR.

- Verify the SWEA door has been open for at least 1 hour.

Verify_____________

- Verify a high voltage enable plug or enable GSE in installed in SWEA-J2

Enable Type___________

- Enable MCPHV by sending the following two commands separated by no more than 15 seconds.

/SWEAArmMCPHV

/SWEAMCPHVOn

Verify no commanding errors reported on the C&T GSE

Verify___________

- Verify that the SWEA MCPHV state is On (on the C&T GSE ISWEASTEDDig display, MCPHVENB = 1).

Verify___________

- Record the following high voltage value from the C&T GSE SWEA/STE-D Housekeeping display:

ISWEAMCP (0V)___________

### SWEA MCP HV Ramp Up

Ramp the SWEA MCP HV up to its nominal level through a few steps.  Record the indicated data at each step.  Note that the IDPU will ramp the voltage up to the desired setting at ~100V/second.  Wait for ramping to complete before taking the measurement.

| Command | Nominal Voltage | Instrument Current | ISWEAMCP (C&T GSE) | Moment N (SWEA GSE) |
| --- | --- | --- | --- | --- |
| /SetSWEAMCP 0 | 0 |  |  |  |
| /SetSWEAMCP 36 | 500 |  |  |  |
| /SetSWEAMCP 73 | 1000 |  |  |  |
| /SetSWEAMCP 109 | 1500 |  |  |  |
| /SetSWEAMCP 146 | 2000 |  |  |  |
| /SetSWEAMCP 182 | 2500 |  |  |  |
| /SetSWEAMCP 190 | 2600 |  |  |  |
| /SetSWEAMCP 197 | 2700 |  |  |  |
| /SetSWEAMCP 204 | 2800 |  |  |  |
| /SetSWEAMCP 212 | 2900 |  |  |  |
| /SetSWEAMCP 220 | 3000 |  |  |  |
| /SetSWEAMCP 224 | 3050 |  |  |  |
| /SetSWEAMCP 228 | 3100 |  |  |  |
| /SetSWEAMCP 232 | 3150 |  |  |  |

### SWEA V0 Setting

If a non-zero V0 value is desired send

/SWEAManualV0

/SetSWEAManualV0 <value>

Value is –Voltage*256/25 (so a value of 10 is ~0.98V)

V0 Setting_____________

### Run SWEA Calibration Sequence

Start the SWEA Calibration sequence to test SWEA MCP setting:

/StartCmdSeq 14

Verify___________

## Final Configuration

The system is now ready for SEP and/or PLASTIC power up.  If desired, adjust the configuration as follows:

- Enable SEP Crash Recovery (after SEP is operational) if desired:

/EnableCmdSequence 12	Y/N__________

- Enable SWEA Recovery Sequence if desired:

/EnableCmdSequence 18	Y/N__________

- Enable SWEA Recovery Polling if desired:

/ExecutePolled 0xEF00	Y/N__________

- Enable PLASTIC power sequences if desired (check with PLASTIC)

/EnableCmdSequence 6	Y/N__________

/EnableCmdSequence 7	Y/N__________

- Enable PLASTIC sequences if desired (check with PLASTIC)

/EnableCmdSequence 20	Y/N__________

/EnableCmdSequence 21	Y/N__________

/EnableCmdSequence 22	Y/N__________

/EnableCmdSequence 24	Y/N__________

/EnableCmdSequence 27	Y/N__________

/EnableCmdSequence 29	Y/N__________

- Start PLASTIC startup sequence if desired (check with PLASTIC)

/StartCmdSequence 21	Y/N__________

- Record final IcmdSeqMask:	__________________

- Set SWAVES burst trigger inputs: Start SWAVESBurstDefault1.  Verify that IBurstCurCriteria is <255	IBurstCurCriteria___________

- Disable IMPACT “disable science on SSR full” system if desired (causes trouble to Real Time data if they are not keeping SSR empty during commissioning): /SetIMPACTSSRLimit 101 	Y/N__________

- Disable PLASTIC “disable science on SSR full” system if desired (causes trouble to Real Time data if they are not keeping SSR empty during commissioning): /SetPLASTICSSRLimit 101 	Y/N__________

- If desired, enable SWEA spectra data.  Send /SetBurstRate 492 to reduce the burst telemetry rate a bit to make room for spectra.  The send /SetSWEASpecInterval 10 (one 48-energy spectra every 10 seconds), and /SWEAEnableSpecTlm 	Y/N__________

- If desired, set the default IMPACT burst trigger weights and SWEA criteria energy band.  Send Start IMPACTBurstDefault2. 	Y/N__________

- If desired, disable the PLASTIC burst trigger.   Send: /SetBurstWeightPLASTIC 0x00

/SetBurstTableID 3 	Y/N__________

- If desired, set the Slow burst mode (SWEA/MAG/No STE).   Send:

/startBurstMode4.cmd

Y/N__________

- Disable SWEA Moments, PAD telemetry, optionally increase Dist rate:

/SWEADisableMOMTlm

/SWEADisablePADTlm

Optionally (as of 12/2013, we do not do this):

/SetSWEADistInt 20 	Y/N__________

- Dump the parameter tables: /DumpSystemTable 	Y/N__________

- Set to nominal housekeeping rate: /SetHKPRate 60 	Y/N__________

Completion Time_____________________

# Power Off

Record Start Time____________________

- This procedure powers off the IMPACT Boom suite.  SEP and PLASTIC should be powered off first.

Verify SEP Powered Off_________

Verify PLASTIC Powered Off_________

- Set the Housekeeping rate to every 2 seconds to speed up the response on the following tasks.  /SetHKPRate 2.  Verify housekeeping packets are now coming every 2 seconds

Verify_________

## Power off SWEA HV

- Ramp down the SWEA MCP HV in the following steps.  After each step wait for the MCP voltage to stabilize before proceeding to the next step.

| Command | Nominal Voltage | ISWEAMCP (C&T GSE) |
| --- | --- | --- |
| /SetSWEAMCP 182 | 2500 |  |
| /SetSWEAMCP 146 | 2000 |  |
| /SetSWEAMCP 109 | 1500 |  |
| /SetSWEAMCP 73 | 1000 |  |
| /SetSWEAMCP 36 | 500 |  |
| /SetSWEAMCP 0 | 0 |  |

- Power off the high voltage supplies: Send /SWEAMCPHVOff, /SWEANRHVOff.  Verify MCPHVENB and NRHVENB are off (0) in ISWEASTEDDig on the C&T GSE.

Verify___________

## STE Door Close

- If at ambient conditions (not vacuum, below +40C), send the following command to adjust the door timing:  start STEDoorAir.  If  in vacuum with STE temperatures below –35C do:  start STEDoorVacCold.  If in vacuum and between –35C and +20C do: start STEDoorVacWarm.  If in vacuum and above between +20C and +40C do: start STEDoorVacHot

STE-U temperature___________C

STE-D Temperature___________C

Script:__________________

### STE-U Door Close

- The STE-U door is normally closed at this point in the mission. However, if ISTEUCovState is OPEN, then proceed.

- Command the door closed.  Send the command  /STEUDoorClose.  Verify by inspection that the cover closes:

Verify___________

- If the GSE indicates an STE AFE trip (STE housekeeping goes blue), send /STEUAFEOn

Verify___________

- Record the STEU Door housekeeping.

ISTEUDoorCount___________

ISTEUDoorStart___________

ISTEUDoorDone___________

ISTEUCovStat___________

ISTEUCovSW___________

- Verify that the ISTEUDoorCount increases by 1, the start and done times are in limit (not red/yellow), ISTEUCovStat is now CLOSED, and ISTEUCovSW is now OFF.

Verify___________

### STE-D Door Close

- Unlike the STE-U door, the STE-D door is normally open so this section should usually be executed.

- Command the door closed.  Send the command  /STEDDoorClose.  Verify by inspection that the cover closes:

Verify___________

- If the GSE indicates an STE AFE trip (STE housekeeping goes blue), send /STEDAFEOn

Verify___________

- Record the STEU Door housekeeping.

ISTEDDoorCount___________

ISTEDDoorStart___________

ISTEDDoorDone___________

ISTEDCovStat___________

ISTEDCovSW___________

- Verify that the ISTEDDoorCount increases by 1, the start and done times are in limit (not red/yellow), ISTEDCovStat is now CLOSED, and ISTEDCovSW is now OFF.

Verify___________

## SWEA Power Off

- Request the MOC power off SWEA.

Verify SWEA Power Down Status Flag set__________

Verify SWEA Power shut off 90 seconds later__________

Verify SWEA Interface Disabled__________

Verify SWEA Survival Heater powered on__________

Record SWEA Survival Heater Current__________

## IDPU Power Off

- Request the MOC power off IDPU.

Verify IDPU Power Down Status Flag set__________

Verify IDPU Power shut off 90 seconds later__________

Verify MAG Heater power ON__________

Record MAG Heater Current__________

Completion Time_____________________